# FQCT Server

User Guide — LCD Defect Detection for AC Remote Production QC

System:  FQCT Inference Server     Model:  EfficientNet-B0 (dts_p2v2_best.pth)     Version:  2.0     Last updated:  July 2026

──────────────────────────────────────────────────────────────────────────────────────────

# 1.  What This System Does

The FQCT Inference Server inspects AC remote LCD displays during the factory DTS (Display Test Sequence). A short video of the LCD in its all-segments-on state is submitted to the server; the server analyses it using a trained deep learning model and returns a PASS or FAIL verdict within approximately 5–10 seconds.

The system runs on a central GPU machine on the factory LAN and can handle up to 10 simultaneous inspection jobs — one per test unit.

# 2.  Accessing the Dashboard

Open a web browser and go to:

http://tangentthoughttech.com:8080

If you are on the same LAN as the server, use the server's local IP address instead:

http://<SERVER_LAN_IP>:8080

You will see the FQCT Inference Server dashboard with three sections:

| Section | What it shows |
| --- | --- |
| Submit Video for Inspection | Upload form for manual video testing |
| Recent Jobs | History of all past inspection results |
| Config | Live configuration values (for admin use) |

Note:  If you are accessing from outside the factory LAN, see the Troubleshooting section for the SSH tunnel command.

# 3.  Submitting a Video for Inspection

## Steps

Open the dashboard at http://tangentthoughttech.com:8080

In the Submit Video for Inspection box:

Click Submit

After 5–10 seconds the result appears directly below the form.

Job ID — leave blank to auto-generate, or type a meaningful name (e.g. unit03_batch01)

Video — click Choose File and select your .mp4 video file

## Video format requirements

| Property | Requirement |
| --- | --- |
| Format | MP4 (.mp4) or AVI (.avi) |
| Frame rate | 5 FPS or 12 FPS (both supported) |
| Resolution | Any (1080p recommended) |
| Content | Must contain the LCD splash state (all segments on) |
| Duration | 10–30 seconds is ideal |

Note:  The video should capture the DTS all-segments-on phase — the moment when every digit, icon, and segment on the LCD is illuminated simultaneously.

# 4.  Reading the Result

After processing you will see a result like this:

PASS
Job: job_1784322273633_ghs8  |  P2 prob: 94.0%  |  P2 frames: 49/60  |  Inference: 4596ms

## Verdict

| Verdict | Colour | Meaning |
| --- | --- | --- |
| PASS | Green | LCD passed inspection — all segments detected above threshold |
| FAIL | Red | LCD failed inspection — one or more segments below threshold |

## Metrics explained

| Field | Meaning |
| --- | --- |
| P2 prob | Confidence score from the model. Higher = more confident PASS. Values above 50% → PASS. Example: 94.0% means very high confidence. |
| P2 frames | How many frames were analysed out of the total sampled. Example: 49/60 means 49 frames qualified as Phase-2 out of 60 frames checked. |
| Inference | Time taken to process the video (milliseconds). Typically 4,000–9,000 ms. |

## Segment table

Below the verdict you will see a table of the five LCD zones that were checked:

| Segment | What it covers |
| --- | --- |
| left_clock | Left clock time display (TIME OFF digits) |
| right_clock | Right clock time display (TIME ON digits) |
| center_88 | Large centre temperature digits (88) |
| signal_bars | Signal / fan-speed bar indicators |
| bottom_88888 | Bottom five-digit strip (88888) |

Each row shows:

| Column | Meaning |
| --- | --- |
| Coverage | Fraction of the zone that is lit (0.0 – 1.0). A perfect all-on display is typically 0.35 – 0.85 depending on zone. |
| Min | Minimum coverage required for that zone to pass. |
| Status | OK (green) if coverage ≥ min, FAIL (red) if below. |

## Demo result (from live test — 18 Jul 2026)

The following result was captured from a live 12 FPS test video (video@12FPS_20260714_143041.mp4) — all five zones passed:

| Segment | Coverage | Min | Status |
| --- | --- | --- | --- |
| left_clock | 0.399 | 0.38 | OK |
| right_clock | 0.308 | 0.25 | OK |
| center_88 | 0.677 | 0.60 | OK |
| signal_bars | 0.760 | 0.35 | OK |
| bottom_88888 | 0.834 | 0.55 | OK |

Result: PASS  ·  P2 prob 94.0%  ·  49/60 Phase-2 frames  ·  4,596 ms inference time.

# 5.  The Overlay Image

Directly below the segment table, an annotated image of the best Phase-2 frame from the video is displayed. This is the clearest frame the model selected for inspection.

┌─────────────────────────────────────┐
│  DTS v2: PASS                       │  ← Verdict banner (green=PASS, red=FAIL)
│  conf=93.9%  P2=60fr (49ok/11fail)  │  ← Confidence and frame count
│                                     │
│   [LCD image with coloured boxes]   │  ← Actual LCD frame from the video
│                                     │
│  Boxes:  green = segment passed     │
│          red   = segment failed     │
└─────────────────────────────────────┘

The overlay shows exactly which part of the LCD was checked and whether it passed. A red box on any zone indicates that zone had insufficient segment coverage in the video.

Note:  You can also open the overlay image in full size by clicking the overlay link in the Recent Jobs table.

# 6.  Recent Jobs Table

The dashboard automatically refreshes every 5 seconds and shows the last 30 jobs.

| Column | Meaning |
| --- | --- |
| Job ID | Unique identifier for this inspection run |
| Status | pending → processing → done or error |
| Verdict | PASS (green) / FAIL (red) once done |
| P2 prob | Model confidence (higher = more certain PASS) |
| P2 frames | Frames passing Phase-2 gate / total sampled |
| Inference (ms) | Processing time in milliseconds |
| Created | Date and time the job was submitted |
| Overlay | Link to open the annotated LCD frame image |
| Error | Error message if the job failed (e.g. no_phase2_frame_detected) |

## Status meanings

| Status | Colour | Meaning |
| --- | --- | --- |
| pending | White | Job is in the queue, waiting for a free worker |
| processing | Orange | Inference is running now |
| done | Green | Inference complete — verdict is available |
| error | Red | Something went wrong — check the Error column |

# 7.  Server Statistics

At the top of the page, five live counters show the current server state. A healthy server at idle shows 0 Pending and 0 Processing.

| Card | Meaning |
| --- | --- |
| Pending | Jobs queued but not yet started |
| Processing | Jobs currently running (max 10) |
| Done (all time) | Total completed inspections since server started |
| Errors | Total failed jobs |
| Avg inference (ms) | Average processing time over the last 100 jobs |

# 8.  Adjusting Configuration

At the bottom of the dashboard is the Config panel showing the live server settings. To change a value without restarting the server:

Type the key in dot-notation in the first field, e.g. model.prob_threshold

Type the new value in the second field, e.g. 0.6

Click Update — the change takes effect immediately for all new jobs

## Key settings

| Key | Default | Effect of changing |
| --- | --- | --- |
| model.prob_threshold | 0.5 | Lower → more lenient (more PASS). Raise → stricter (more FAIL). |
| video.frame_step | 5 | Lower = more frames analysed = slower but more accurate. |
| video.dark_thresh | 110 | Adjust if lighting conditions change (pixel brightness cutoff). |
| phase2.digit_score_min | 0.35 | Min segment fill to qualify a frame as Phase-2. |

Note:  Changes are written to disk and persist across server restarts.

# 9.  Using the API (Automated / CM4 Units)

The server exposes a simple HTTP API for automated test units. All requests go to http://<SERVER_IP>:8080.

## Submit a video (fire-and-forget)

curl -X POST http://<SERVER_IP>:8080/inspect_queue \
     -F "video=@/path/to/dts_test.mp4" \
     -F "job_id=unit03_$(date +%s)"

Response:

{"job_id": "unit03_1752000000", "status": "queued"}

## Poll for result (after ~20 s IR test window)

curl "http://<SERVER_IP>:8080/inference_result?job_id=unit03_1752000000"

Response when done:

{
  "job_id": "unit03_1752000000",
  "status": "done",
  "result": {
    "passed": true,
    "verdict": "PASS",
    "median_p2_prob": 0.9395,
    "p2_frames": 49,
    "p2_pass": 49,
    "p2_fail": 0,
    "inference_ms": 4596.0
  }
}

Use result.passed (true / false) for your automated go/no-go decision.

## All endpoints

| Method | URL | Purpose |
| --- | --- | --- |
| POST | /inspect_queue | Submit video for inspection |
| GET | /inference_result?job_id=<id> | Get result for a job |
| GET | /jobs/<id>/overlay | Download overlay JPEG |
| GET | /health | Server liveness check |
| GET | /status | Queue depth and statistics |
| GET | /api/jobs?limit=50 | Recent job list (JSON) |
| GET | /api/config | Current config (JSON) |
| POST | /api/config | Update a config key |
| GET | / | Web dashboard |

# 10.  Troubleshooting

## "no_phase2_frame_detected" error

The server could not find any frame where all LCD segments are fully on.

| Cause | Fix |
| --- | --- |
| Video recorded before the DTS splash started | Start recording 2–3 seconds before the splash state begins |
| Video is too short | Ensure recording covers at least 5 seconds of the all-on state |
| Camera glare on LCD | Reposition camera or adjust lighting to reduce reflections |
| phase2.digit_score_min too strict | Lower slightly via Config panel (e.g. 0.28) |

## Job stuck in "processing" for more than 60 seconds

Check the server statistics — if Processing count is at 10, all workers are busy. Wait.

If stuck for more than 2 minutes, contact the system admin to restart the server.

## FAIL result on a unit that looks visually fine

Open the overlay image for that job — identify which segment zone is red.

Check whether the camera was properly positioned during recording.

If coverage values are close to the minimum, model.prob_threshold may need adjustment.

Retry with a fresh recording if the original video had motion blur or poor focus.

## Dashboard shows no jobs / page will not load

The server may be down. Run a health check:

curl http://<SERVER_IP>:8080/health

If this fails, ask the admin to restart the server:

# Admin: SSH to server and restart
ssh om@tangentthoughttech.com
kill $(lsof -ti:8080) 2>/dev/null; sleep 1
cd /home/om/src/fqct_server
nohup python3 -m uvicorn server.app:app --host 0.0.0.0 --port 8080 \
    --config config/fqct_server.yaml > server.log 2>&1 &

## Cannot reach the dashboard from outside the LAN

Use an SSH tunnel — run this on your local machine, then open http://localhost:8080:

sshpass -p 'useme123' ssh -N -L 8080:localhost:8080 om@tangentthoughttech.com

# 11.  Quick Reference

## Dashboard URL

http://tangentthoughttech.com:8080

## Typical workflow

1.  Open the dashboard in a browser

2.  Enter a Job ID (or leave blank for auto-generated)

3.  Click "Choose File" and select the DTS video (.mp4)

4.  Click Submit

5.  Wait 5–10 seconds

6.  Read the verdict: PASS (green) or FAIL (red)

7.  If FAIL: click the overlay link to see which segment zone failed

## Result interpretation at a glance

| P2 prob | P2 frames ratio | What it means |
| --- | --- | --- |
| > 90 % | > 70 % | Strong PASS — all segments clearly on |
| 60–90 % | 50–70 % | Confident PASS — minor variation in some frames |
| 50–60 % | Any | Marginal PASS — re-test recommended |
| < 50 % | Any | FAIL — segment(s) missing or dim |

## Contact / Support

| Item | Detail |
| --- | --- |
| Contact | Om Kathalkar — CVIT Lab, IIIT Hyderabad |
| Project | IP4R / FQCT |
| Repository | ~/phd-efficient-inference/IP4R/ |
| Server | om@tangentthoughttech.com port 8080 |
| Email | omkathalkar319@gmail.com |

──────────────────────────────────────────────────────────────────────────────────────────

This guide covers FQCT Server v2.0 (EfficientNet-B0, Phase-2-gated). Last updated July 2026.